QUESTION FOUR  Resistors in Series and Parallel

Two circuits were set up, one in series the other in parallel. All the resistors have the same resistance. The purpose was to investigate the use of energy in the two types of circuit.

20 V

20 V

  - In the parallel circuit, the energy supplied by the power supply per minute is 5.6 kJ.

- Calculate the power output of the power supply.

- Explain why the power consumed by each of the resistors is half the power output of the supply.

- Show that the resistance of each resistor is 8.6 .

  - Calculate the power consumed by each of the resistors in the series circuit and hence calculate the energy supplied by the supply per minute.

  - If the purpose of the resistors is to supply heat, explain which of the two circuits is more efficient.